# UETDRRC002 Install and maintain low voltage underground services in a very remote community

Release: 1

UETDRRC002 Install and maintain low voltage underground services in a very remote community

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package Release 2.0.

# Application

This unit involves the skills and knowledge required to install and maintain low voltage (LV) underground services in very remote communities in the electricity supply industry (ESI). It includes the installation and maintenance of LV underground services and associated equipment between the customer’s connection point and the network point of supply. It also includes testing and commissioning, identifying and rectifying faults, and the repair and replacement of service cables. All installation and maintenance of LV underground service cables in very remote communities must be undertaken in an isolated environment other than for testing purposes. The application of the skills and knowledge described in this unit may require a licence/registration to practice in the workplace. Other conditions may also apply under state and territory legislative and regulatory licensing requirements which must be confirmed prior to commencing this unit.

# Pre-requisite Unit

UEECD0007 Apply work health and safety regulations, codes and practices in the workplace UEECD0046 Solve problems in single path circuits UEECD0051 Use drawings, diagrams, schedules, standards, codes and specifications UETDREL001 Apply environmental requirements UETDREL004 Operate plant and equipment in the vicinity of live electrical apparatus UETDREL005 Work safely in the vicinity of live electrical apparatus UETDRRC006 Perform low voltage electricity network switching in a very remote community UETDRRC007 Solve problems in electrical network apparatus in a very remote community UETDRRC008 Solve problems in low voltage electrical network circuits in a very remote community

# Competency Field

Remote Community

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare to install and maintain LV underground services | 1.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are referred to and confirmed |
|  |  | 1.2 | Work plan is obtained and confirmed in accordance with workplace requirements |
|  |  | 1.3 | Materials required for work are determined and obtained in accordance with work plan and workplace requirements |
|  |  | 1.4 | Work is prioritised and sequenced for completion in accordance with workplace requirements |
|  |  | 1.5 | Hazards are identified, risks assessed and control measures identified and applied |
|  |  | 1.6 | Underground electrical distribution network is verified as isolated and earthed/short circuited in accordance with workplace requirements |
|  |  | 1.7 | Work permits are received and signed in accordance with workplace requirements |
|  |  | 1.8 | Plant, tools, equipment, and personal protective equipment (PPE) required for work are determined, obtained and confirmed in working order |
|  |  | 1.9 | Worksite is prepared in accordance with work plan and workplace requirements |
|  |  | 1.10 | Traffic management plan is confirmed as being applied in accordance with workplace requirements |
| 2 | Carry out installation and maintenance of LV underground services | 2.1 | Legislation, regulations, standards, codes of practice and organisational workplace requirements for the work to be performed are applied and monitored |
|  |  | 2.2 | Lifting, climbing, working at heights and the use of plant, tools, equipment and PPE are carried out in accordance with workplace requirements |
|  |  | 2.3 | Hazard control measures are monitored in accordance with workplace requirements |
|  |  | 2.4 | Installation and maintenance of LV underground service is completed in accordance with work plan and workplace requirements |
|  |  | 2.5 | Earthing and short circuiting equipment is removed, permit signed off and underground electrical distribution network is re-energised in accordance with workplace requirements |
|  |  | 2.6 | LV underground services are tested in accordance with work plan and workplace requirements |
|  |  | 2.7 | Incidents or unplanned events are responded to in accordance with workplace requirements |
|  |  | 2.8 | Quality checks of work are undertaken in accordance with workplace requirements |
| 3 | Complete work and documentation | 3.1 | Completed work is checked for compliance against the work plan and workplace requirements |
|  |  | 3.2 | Incidents or unplanned events are reported in accordance with workplace requirements |
|  |  | 3.3 | Worksite is rehabilitated, cleaned and made safe in accordance with workplace requirements |
|  |  | 3.4 | Plant, tools and equipment are cleaned, checked and returned to storage in accordance with workplace requirements |
|  |  | 3.5 | Surplus materials are returned to storage or disposed of in accordance with workplace requirements |
|  |  | 3.6 | Work records, reports and documentation are completed, and appropriate personnel notified in accordance with workplace requirements |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the UET Transmission, Distribution and Rail Sector Training Package Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UETTDRIS37 Install and maintain low voltage services in remote communities (underground).

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7