# Assessment Requirements for UETDRVC007 Control vegetation using pruning techniques

Release: 1

Assessment Requirements for UETDRVC007 Control vegetation using pruning techniques

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package Release 2.0.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least two separate occasions and include:

applying relevant legislation, regulations, standards, codes of practice and organisational workplace requirements, including: work health and safety (WHS)/occupational health and safety (OHS) identifying hazards and assessing risks identifying, applying and monitoring control measures obtaining, inspecting and using relevant personal protective equipment (PPE) pruning vegetation in accordance with workplace requirements, including: using techniques for shaping and forming vegetation controlling/managing epicormic regrowth using appropriate cutting techniques for branch removal applying the principles of compartmentalization of decay in trees (CODIT) determining the correct location for the final cut using species appropriate pruning techniques using tools and equipment safely performing pre-operational checks on tools and equipment using correct cutting angle in accordance with workplace requirements, including: branch bark ridge stem bark ridge visible branch collar determining suitable growth points dealing with an unplanned event on at least one (1) occasion completing relevant work records, reports and documentation.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

relevant legislation, regulations, standards, codes of practice industry and workplace requirements, including: WHS/OHS hazard, risk assessment and risk control requirements, including potential hazards types and application of PPE safe use of tools and equipment events constituting an unplanned event or incident procedures for responding to an unplanned event or incident pruning principles and techniques, including: shaping, forming and controlling epicormic regrowth preventing disease and damage (CODIT) promoting tree health effects of incorrect pruning tree physiology (basic), including: photosynthesis transpiration respiration plant development and growth (including epicormic growth) biosecurity practices for pruning, including: sterilisation of clothing, tools and equipment management of debris removal pruning tools and equipment, including: inspection care and maintenance storage and transport pruning techniques, including: formative preventative natural target pruning sequence (cutting techniques) for branch removal crown modification directional pruning.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations where it is appropriate to do so. Where this is not appropriate, assessment must occur in simulated conditions involving realistic and authentic activities that replicate operational workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: relevant and appropriate materials, tools, equipment and PPE currently used in industry applicable documentation, including workplace requirements, equipment specifications, regulations, codes of practice and operation manuals.

# Links

Companion Volume Implementation Guides are found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7