# UETTDRRT23 Install rail traction bonds

Release: 1

UETTDRRT23 Install rail traction bonds

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package.

# Application

This unit involves the skills and knowledge required to install rail traction bonds in the electricity supply industry (ESI). It includes the installation of temporary and permanent traction bonds, bonding cables and using safe working practices on or about the running line/track. It also includes isolating systems and circuits for safe working in accordance with work plans, identifying and implementing traffic management plans, issuing/accepting electrical permits, checks to confirm bonds, bonding cables and associated hardware have been correctly installed and are in a safe condition to test prior to putting into service, undertaking pre-commissioning tests and updating installation data and relevant quality assurance documentation. The application of the skills and knowledge described in this unit may require a licence/registration to practice in the workplace. Other conditions may apply under state and territory legislative and regulatory licensing requirements which must be confirmed prior to commencing this unit.

# Pre-requisite Unit

Common Unit Group UEENEEE101A Apply Occupational Health and Safety regulations, codes and practices in the workplace UEENEEE102A Fabricate, assemble and dismantle utilities industry components UEENEEE104A Solve problems in d.c. circuits UEENEEE105A Fix and secure electrotechnology equipment UEENEEE107A Use drawings, diagrams, schedules, standards, codes and specifications UEENEEG101A Solve problems in electromagnetic devices and related circuits UEENEEG102A Solve problems in low voltage a.c. circuits UETTDREL11 Apply sustainable energy and environmental procedures UETTDREL12 Operate plant and equipment near live electrical conductors and apparatus UETTDREL16 Working safely near live electrical apparatus UETTDRIS52 Install and maintain poles, structures and associated hardware UETTDRIS54 Install and maintain poles, structures and overhead conductors and cables

# Competency Field

Rail Traction

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Prepare to install traction bonds | 1.1 | Work schedules, construction plans, drawings, workplace procedures and material lists are received and confirmed |
|  |  | 1.2 | Job requirements and workplace procedures are identified and communicated with relevant personnel |
|  |  | 1.3 | Work health and safety (WHS)/occupational health and safety (OHS), environmental and workplace procedures for the installation of traction bonds are obtained and confirmed for the purposes of the work to be performed |
|  |  | 1.4 | Work is prioritised and sequenced for completion within acceptable timeframes following consultation with relevant personnel and in accordance with workplace procedures |
|  |  | 1.5 | Hazards are identified, WHS/OHS risks assessed and control measures identified in accordance with workplace procedures |
|  |  | 1.6 | Relevant work permits are obtained to access rail corridor, understood and signed in accordance with workplace procedures |
|  |  | 1.7 | Plant, equipment, tools and personal protective equipment (PPE) required for work are identified, obtained and confirmed in working order |
|  |  | 1.8 | Worksite is prepared to minimise risk, damage to property, commerce and individuals in accordance with track protection officer and/or network control officer and workplace procedures |
|  |  | 1.9 | Traffic management plan is confirmed as being applied in accordance with workplace procedures |
|  |  | 1.10 | Environmental constraints applicable to work are identified and control measures applied |
| 2 | Perform installation of traction bonds | 2.1 | WHS/OHS, sustainable energy and environmental principles and practices are monitored and followed to reduce incidents in accordance with workplace procedures |
|  |  | 2.2 | Requirements for lifting and/or climbing and/or working at heights and the use of power tools/equipment are followed in accordance with workplace procedures |
|  |  | 2.3 | Systems and circuits are checked, isolated and proved safe to work on in accordance with workplace procedures |
|  |  | 2.4 | Installation of traction bonds and bonding cables are completed in agreed timeframes, to quality standards and with minimum waste in accordance with workplace procedures |
|  |  | 2.5 | Cable and surrounds, including rail and other surfaces, are prepared for jointing and terminating in accordance with workplace procedures |
|  |  | 2.6 | Traction bonds are installed and tested in accordance with job requirements and workplace procedures |
|  |  | 2.7 | Unplanned events are responded to in accordance with workplace procedures |
|  |  | 2.8 | Quality checks of work are undertaken in accordance with workplace procedures |
| 3 | Complete installation of traction bonds | 3.1 | Completed work is checked and tested against works schedule, construction plans and drawings for compliance and anomalies reported in accordance with workplace procedures |
|  |  | 3.2 | Worksite is rehabilitated, cleaned up and made safe in accordance with workplace procedures |
|  |  | 3.3 | Tools, equipment and surplus resources and materials are, where appropriate, cleaned, checked and returned to storage in accordance with workplace procedures |
|  |  | 3.4 | Relevant work permits are signed off and, the system that has undergone the installation of a traction bond/s is returned to service in accordance with workplace procedures |
|  |  | 3.5 | Work records, reports and/or documentation are completed in accordance with workplace procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UETTDRRT23A Install rail traction bonds.

# Links

UET Training Package Companion Volume Implementation Guide is found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7