# UETTDRSO43 Coordinate low voltage distribution network demand

Release: 1

UETTDRSO43 Coordinate low voltage distribution network demand

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package.

# Application

This unit covers the coordination of the switching of low voltage (LV) distribution network components with due regard to the loadings and prevailing network constraints, including scheduling of generators, Volt Ampere Reactive (VAR) compensators, load shedding and non-essential loads in response to National Electricity Market Management Company (NEMMCO) or network requirements. It also includes voltage control equipment. The application of the skills and knowledge described in this unit may require a licence/registration to practice in the workplace subject to regulations for undertaking of electrical work. Other conditions may apply under state and territory legislative and regulatory licencing requirements which must be confirmed prior to commencing this unit.

# Pre-requisite Unit

Common Unit Group UEENEED104A Use engineering applications software on personal computers UEENEEE101A Apply Occupational Health and Safety regulations, codes and practices in the workplace UEENEEE102A Fabricate, assemble and dismantle utilities industry components UEENEEE104A Solve problems in d.c. circuits UEENEEE107A Use drawings, diagrams, schedules, standards, codes and specifications UEENEEE124A Compile and produce an energy sector detailed report UEENEEE125A Provide engineering solutions for problems in complex multiple path circuits UEENEEE126A Provide solutions to basic engineering computational problems UEENEEG101A Solve problems in electromagnetic devices and related circuits UEENEEG102A Solve problems in low voltage a.c. circuits UEENEEG149A Provide engineering solutions to problems in complex polyphase power circuits UETTDREL11 Apply sustainable energy and environmental procedures UETTDREL16 Working safely near live electrical apparatus UETTDRIS62 Implement and monitor the power system organisational WHS/OHS policies, procedures and programs UETTDRIS63 Implement & monitor power system environmental & sustainable energy management policies & procedures UETTDRSO36 Develop low voltage distribution switching programs UETTDRSO39 Coordinate low voltage distribution networks

# Competency Field

System Operations

# Unit Sector

Not applicable.

# Elements and Performance Criteria

| ELEMENTS |  | PERFORMANCE CRITERIA |  |
| --- | --- | --- | --- |
| Elements describe the essential outcomes. |  | Performance criteria describe the performance needed to demonstrate achievement of the element. |  |
| 1 | Plan for the coordination of LV distribution network demand | 1.1 | Work health and safety (WHS)/occupational health and safety (OHS) practices/procedures and environmental and sustainable energy procedures, which may influence the coordination of LV distribution network demand systems, are reviewed and determined |
|  |  | 1.2 | Purpose of the coordination of the network demand is established after data is analysed and expected outcomes of the work are confirmed with appropriate personnel |
|  |  | 1.3 | Organisational established procedures on policies and specifications for the coordination of the network demand are obtained or established with appropriate personnel |
|  |  | 1.4 | Testing procedures are discussed with/directed to appropriate personnel in order to ascertain the project brief |
|  |  | 1.5 | Testing parameters are established from organisational established procedures on policies and specifications |
|  |  | 1.6 | Equipment/tools and personal protective equipment (PPE) are selected based on specified performance criteria and established procedures |
|  |  | 1.7 | Work roles and tasks are allocated according to requirements and individual competencies |
|  |  | 1.8 | Work is prioritised and sequenced for the most efficient/effective outcome, completed within an acceptable timeframe, to a quality standard and in accordance with established procedures |
|  |  | 1.9 | Liaison and communication issues with others/authorised personnel, authorities, clients and land owners are resolved and activities coordinated to carry out work |
|  |  | 1.10 | Risk control measures are identified, prioritised and evaluated against the work schedule |
|  |  | 1.11 | Relevant work permits are secured to coordinate the performance of work according to requirements and/or established procedures |
| 2 | Carry out the coordination of LV distribution network demand | 2.1 | Circuit/systems modelling are used to evaluate alternative proposals in accordance with established procedures |
|  |  | 2.2 | WHS/OHS and sustainable energy principles, functionality and practices to reduce the incidents of accidents and minimise waste are incorporated into the project in accordance with requirements and/or established procedures |
|  |  | 2.3 | Coordination decisions are made on the basis of safety and effective outcomes according to requirements and/or established procedures |
|  |  | 2.4 | Mathematical and/or engineering models of the coordination process are used to analyse the effectiveness of the finished project in accordance with requirements and established procedures |
|  |  | 2.5 | Technical advice is given regarding potential hazards, safety risks and control measures so that monitoring and preventative action can be undertaken and/or appropriate authorities consulted, where necessary, in accordance with requirements and established procedures |
|  |  | 2.6 | Essential knowledge and associated skills are applied to analyse specific data and compare it with compliance specifications to ensure completion of the project within an agreed timeframe according to requirements |
|  |  | 2.7 | Testing of the coordination process is undertaken according to requirements and established procedures |
|  |  | 2.8 | Work teams/groups are arranged/coordinated/evaluated to ensure planned goals are met according to established procedures |
|  |  | 2.9 | Solutions to non-routine problems are identified and actioned, using acquired essential knowledge and associated skills, according to requirements |
|  |  | 2.10 | Quality of work is monitored against personal performance agreement and/or established organisational and professional standards |
|  |  | 2.11 | Strategic plans are developed incorporating organisation initiatives in accordance with established procedures |
| 3 | Complete the coordination of LV distribution network demand | 3.1 | Final review of the coordination process is undertaken to ensure they comply with all requirements and include all specifications and documentations needed to complete the project |
|  |  | 3.2 | Appropriate personnel are notified of completion and reports and/or completion documents are finalised/commissioned |
|  |  | 3.3 | Reports and/or completion documents are submitted to relevant personnel/organisations for approval and, where applicable, statutory or regulatory approval |
|  |  | 3.4 | Approved copies of managed LV distribution network demand documents are issues and records are updated in accordance with established procedures |

# Foundation Skills

Foundation skills essential to performance are explicit in the performance criteria of this unit of competency.

# Range of Conditions

Range is restricted to essential operating conditions and any other variables essential to the work environment. Non-essential conditions may be found in the Companion Volume Implementation Guide.

# Unit Mapping Information

This unit replaces and is equivalent to UETTDRSO43A Coordinate low voltage distribution network demand.

# Links

UET Training Package Companion Volume Implementation Guide is found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7

Assessment Requirements for UETTDRSO43 Coordinate low voltage distribution network demand

# Modification History

Release 1. This is the first release of this unit of competency in the UET Transmission, Distribution and Rail Sector Training Package.

# Performance Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria on at least two separate occasions and include:

applying relevant work health and safety (WHS/occupational health and safety (OHS) requirements, including the use of risk control measures applying sustainable energy principles and practices demonstrating at least three (3) coordination of multiple simultaneous system manipulations that encompass the following: low voltage (LV) distribution network LV network manipulation to control loading on equipment transformers with LV windings (if applicable to enterprise equipment) LV busbars LV isolators LV switchgear (applicable to enterprise equipment) coordinating LV distribution network demand including all of the following: writing switching instructions checking and review switching instructions coordinating multiple simultaneous switching instructions confirming line loading preparing and authorising LV distribution switching programs coordinating multiple switching processes coordinating the status of all access permits/authorities on LV network equipment relevant to the scope ensuring network plant operates within design and regulatory requirements on a real time basis applying and administrating of supervisory control and data acquisition (SCADA) or equivalent analysing, diagnosing and reporting of system failure calculating and analysing paralleling conditions on the interconnected LV system allowing safe network access for maintenance activities, including preparing, writing and checking switching sheets to: manage load manage voltage minimise loss maximise system reliability allow safe network access for maintenance activities allow safe network access for construction activities validating fault reports arising from system disturbances dealing with unplanned events on at least one (1) occasion.

# Knowledge Evidence

Evidence required to demonstrate competence in this unit must be relevant to and satisfy all of the requirements of the elements and performance criteria and include knowledge of:

enterprise-specific procedures and work practices relating to managing network demand encompassing: Commonwealth/state/territory and local government legislation, supply authority regulations, standards, codes and/or enterprise requirements applicable to the procedures and work practices relating to managing network demand requirements for the use of demand management manuals, system diagrams/plans and drawings identifying and interpreting enterprise demand management procedures techniques in the applying enterprise demand management procedures effective management and communication encompassing: Commonwealth/state/territory and local government legislation, standards, codes supply authority regulations and/or enterprise requirements applicable to assisting in effective management and communication enterprise operational principles - workplace WHS/OHS enterprise plan, environmental enterprise policies and procedures, industrial relations policies and procedures, and anti-discrimination policies and procedures relationship between the management and employees - methods used to collate and distribute/disseminate information, responsibilities of each member of the work team, staff development activities and legislation requirements with regard to WHS/OHS training, methods of addressing barriers such as literacy and cultural differences, and provisions relating to WHS/OHS issue resolution techniques associated with organisational policies and procedures related to human resources - relevant awards and certified agreements, legislation impacting on people management, and range of support services and expertise available techniques in managing relationships - identifying problems; methods of conflict resolution; methods of consultation, communication, negotiation and mentoring; and strategies for positive feedback techniques in leadership in achieving enterprise strategic and operational plans techniques in managing relationships under stress - stress management methodology used in writing enterprise-specific management reports encompassing: Commonwealth/state/territory and local government legislation, supply authority regulations, standards, codes and/or enterprise requirements applicable to the writing enterprise-specific management reports techniques in researching, collating and analysing information for the report - recording, filing, retrieving systems; and storing and retrieving data from computer systems relationship of management reports to enterprise policies and procedures - enterprise structure and resources, workplace WHS/OHS and risk management enterprise data, financial and operational data, environmental enterprise policies and procedures, industrial relations policies and procedures, and anti-discrimination policies and procedures techniques in writing enterprise-specific management reports - methods used to disseminate information and facilitate enterprise requirements, document proformas, and compliance and legislative requirements to produce effective reports in the appropriate format.

# Assessment Conditions

Assessors must hold credentials specified within the Standards for Registered Training Organisations current at the time of assessment. Assessment must satisfy the Principles of Assessment and Rules of Evidence and all regulatory requirements included within the Standards for Registered Training Organisations current at the time of assessment. Assessment must occur in workplace operational situations where it is appropriate to do so; where this is not appropriate, assessment must occur in simulated conditions involving realistic and authentic activities that replicate operational workplace conditions. Assessment processes and techniques must be appropriate to the language, literacy and numeracy requirements of the work being performed and the needs of the candidate. Resources for assessment must include access to: a range of relevant exercises, case studies and/or other simulations relevant and appropriate materials, tools, facilities, equipment, modelling tools, computerised electrical plant control and monitoring facilities, and personal protective equipment (PPE) currently used in industry applicable documentation, including workplace procedures, relevant industry standards, equipment specifications, regulations, codes of practice, drawings and operation manuals.

# Links

UET Training Package Companion Volume Implementation Guide is found in VETNet - https://vetnet.gov.au/Pages/TrainingDocs.aspx?q=229bace1-b7bc-4653-9300-dffb13ecfad7